Laser Diode Module Power Supply   «LDC-5»

The Power Supply provides pulsed performance of the Laser Diode Modules

Technical parameters

LDM

  - Peak LDM current 	 400 A

  - Max bias current 	 40 A

  - Max LDM voltage 	 400 V

  - LDM average power 	 ≤ 500 W

  - LDM working pulse length	 40 ÷ 3000 µs

  - Repetition rate 	 single shots ÷ CW

  - Instability of the current amplitude 	 ≤ 1 %

  - Power supply of pilot laser 	 5 V  &  200 mA

Peltier Elements 1 & 2

  - Range of the temperature control 	 15 ÷ 90 ºC

  - Discreteness of the temperature control 	 ± 0,1 ºC

  - Max voltage 	 24 V

  - Max current 	 8 A

Additional Temperature Interlock 	 12 ÷ 90 ºC

Photo Detector

  - linear range of the LDM photocurrent measurements 	 0,01 ÷ 160 µA

  - Scale factor of light power/current conversion 	 0 ÷ 100 W/µA

Trigger Delay

  - Tuning range of trigger delay 	 0 ÷ 400 µs

General

  - Power supply 1 (for LDM current) 	 3 ÷ 400 VDC

  - Power supply 2 (for Peltier Element) 	 24 VDC  &  16 A

  - Dimensions 	 120 × 150 × 40 mm3

  - Weight 	 500 g

  - Storage temperature 	 – 30  ÷  + 80 ºC

  - Operating temperature 	 + 5  ÷  + 60 ºC

  - Ambient humidity 	< 98%

The control of the Power Supply is provided by personal computer according to RS232 or USB standard with software VLC3 and VITRONC.

Main Board LDC-5